Computer Networks Assignment 3

# Sharing Files Between Two Computers Using LAN Cable

# PC1:Muhammad Shaheer

# PC2:Abdul Rafay

Name:Muhammad Shaheer.

Roll No:20p-0480

NATIONAL UNIVERSTIY OF COMPUTER AND EMERGING SCIENCES, FAST- PESHAWAR CAMPUS Department of Computer Science & Software Engineering

Sharing Files Between Two Computers Using LAN Cable

Step1:

Connect Both PC’s With LAN Cable Connect both computers to a LAN cable.

Step 2:

Enable Network Sharing on Both PCs

- Goto Control Panel->Network and Internet->Network and Sharing Center->Advanced sharing settings.

- 2- Next, expand All Networks by clicking on the drop-down icon. Here, we need to enable Public Sharing so that the PCs can access files from each other over the LAN cable. To avoid more configuration, just Turn off password protected sharing. By doing so, you enable the other computer to access shared data without providing any credentials. This is not a good security practice by since you are sharing your data with people you trust, you can make this one-time exception. Once you are done with file sharing, make sure you Turn off password protected sharing. Once done, click Save Changes. Just as I said in the beginning, repeat the same steps for the other PC

Step 3: Setup Static IP

1- Goto Control Panel->Network and Internet->Network and Sharing Center.

2- Then On the next window, you will see the active connections which should be

Ethernet, since both PCs are connected with a physical LAN cable. Click on the

Ethernet link. A new dialog box will open, here click on the Properties button.

3- In the next pop-up, select “Internet Protocol Version 4 (TCP/IPv4)”. Now, click on

Properties. This will open another dialogue box.

4- Here, you need to configure the two PCs with different IP settings. On computer 1,

select the option “Use the following IP address.” and, put the following values: IP

Address: 192.168.10.10 Subnet mask: 225.225.225.0 Default gateway: empty Of

course, it’s not necessary you use these IP addresses. You can use any Class A or B IP

address. In case you are not sure what this all means, better stick with this example. On

the second computer, do similar steps, but flip the IP address and Default gateway

values: IP address: 192.168.10.5 Subnet mask: 225.225.225.0 Default gateway: empty

For PC1:

For PC 2:

Step 4: Check Networks on PC’s

Next, open your Window’s File Explorer and click on Network tab at the left side of the

window. You see two PC’s on your computer.

Step 5: Share a folder

1- Next, select the folder you want to share and right-click on it. From the context menu,

navigate to “Give access to” and select the option “Specific People.”

2- In the File Sharing window, choose Everyone from the drop-down menu. Click on the

Add button next to it and finally hit the Share button.

3- Once you have shared it, the next window will show you the network location of the

folder.

4- That’s it. Just go back to the other PC from where you want to access the file, open

the Networks panel and click on the other computer’s name.

5- Here, you will see the folder you just shared. From there, just open the folder and

transfer the files and folders as you normally do. i.e, by copy and paste. Similarly, you

can repeat the same steps from the other computer.

PC 1:

PC 2:

Successfully Transferred…………………………………………..